| Project Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| SMART STUDY DESK ASSISTANT |  |  |  |  |
| Muhammad Usman Abid Nauman Mahmood |  | 25I-6555 25I-6574 |  | B |
| ________________________________________ |  | __________ |  | ____ |  |
| STUDENT NAME |  | ROLL NO |  | SEC |  |
| ____________________________________ |  |  |  |  |
| LAB ENGINEER'S SIGNATURE & DATE |  |  |  |  |
| NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES (NUCES), ISLAMABAD |  |  |  |  |  |

Table of Contents

Abstract	1

Introduction	1

Objectives	1

Components & Materials	2

Software & Libraries	2

Circuit Design	2

Pin Assignment	2

Applications & Future Enhancements	3

Potential Applications	3

Proposed Future Enhancements	3

Conclusion	3

References	4

# Abstract

The Smart Study Desk Assistant is an Arduino Uno-based embedded system designed to enhance student productivity through automated environmental control. The system integrates an HC-SR04 ultrasonic sensor for real-time presence detection, a DHT11 sensor for continuous temperature and humidity monitoring, a white LED as an automatically controlled study light, and a 16×2 I²C liquid-crystal display for live environmental feedback. When the system detects a student seated within 30 cm of the desk, the study light activates and the display shows current temperature and humidity readings. Upon departure, the light switches off and the display updates to show an idle state. The project demonstrates practical application of digital I/O, serial communication protocols (I²C, single-wire), ultrasonic ranging, and embedded control logic using the Arduino platform and the C++ programming language.

# Introduction

Modern students often study for extended periods at a fixed desk, yet the quality of the ambient environment lighting, temperature, and humidity significantly affects cognitive performance, comfort, and focus. Poor lighting causes eye strain; inadequate ventilation or excessive heat reduces concentration. At the same time, lights are frequently left on in empty rooms, wasting energy.

The Smart Study Desk Assistant addresses both concerns simultaneously. It is a compact, low-cost embedded system that:

- Automatically turns the study light ON when a student is present and OFF when they leave, eliminating wasted energy.

- Continuously monitors temperature and humidity, displaying live readings on an LCD so the student can respond to uncomfortable conditions.

- Uses Pomodoro technique for effective study sessions.

- Provides real-time serial output for debugging and data logging via a connected PC.

# Objectives

The specific objectives of this project are:

- Design and build a presence-detection circuit using an HC-SR04 ultrasonic sensor connected to an Arduino Uno.

- Implement automatic study-light control (LED) that responds to presence detection without manual intervention.

- Interface a DHT11 temperature and humidity sensor and retrieve accurate environmental readings.

- Display real-time temperature, humidity, and system status on a 16×2 I²C LCD module.

- Implement robust Arduino firmware in C++ that handles sensor timeouts, DHT11 read errors, and edge cases gracefully.

# Components & Materials

| No. | Component | Qty | Specification | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Arduino Uno R3 | 1× | ATmega328P, 16 MHz, 5 V | Microcontroller / Brain |
| 2 | HC-SR04 Ultrasonic Sensor | 1× | 2–400 cm, ±3 mm | Presence detection |
| 3 | DHT11 Temp & Humidity Sensor | 1× | 0–50 °C, 20–90 %RH | Environmental monitoring |
| 4 | 16×2 LCD with I²C Backpack (PCF8574) | 1× | I²C, addr 0x27 | Live display output |
| 5 | White LED (5 mm) | 1× | 3.2 V forward voltage | Study light |
| 6 | 220 Ω Resistor | 1× | ¼ W, 5% tolerance | LED current limiter |
| 7 | 10 kΩ Resistor | 1× | ¼ W, 5% tolerance | DHT11 DATA pull-up |
| 8 | Half-size Breadboard | 1× | 400 tie-points | Circuit prototyping |
| 9 | Jumper Wires (Male-Male) | ~20× | Various lengths | Wiring / interconnect |
| 10 | USB Cable | 1× | Standard Arduino cable | Power & programming |

## Software & Libraries

| Software / Library | Version / Source | Purpose |
| --- | --- | --- |
| Arduino IDE | 2.x — arduino.cc | Firmware development & upload |
| DHT Sensor Library | Adafruit — Library Manager | DHT11 reading & parsing |
| LiquidCrystal_I2C | F. de Brabander — Library Manager | I²C LCD control |
| Wire.h | Built-in (Arduino core) | I²C hardware abstraction |

# Circuit Design

## Pin Assignment

| Arduino Pin | Direction | Component | Signal |
| --- | --- | --- | --- |
| D2 | Input | DHT11 | Single-wire DATA |
| D9 | Output | HC-SR04 | TRIG |
| D10 | Input | HC-SR04 | ECHO |
| D6, D11 | Output | LED | Anode via 220 Ω |
| A4 (SDA) | Bidirectional | LCD I²C | I²C Data |
| A5 (SCL) | Output | LCD I²C | I²C Clock |
| 5V | Supply | All VCC pins | Power rail |
| GND | Reference | All GND pins | Common ground |

# Applications & Future Enhancements

## Potential Applications

- School and university study rooms - automated energy saving without building-level automation infrastructure.

- Home office desks - smart environment monitoring for remote workers.

- Library individual study carrels - occupancy tracking and comfort monitoring.

- Children's homework stations - parental oversight via serial data logging.

- Shared co-working spaces - real-time occupancy boards using multiple units on a CAN bus or Wi-Fi network.

## Proposed Future Enhancements

| Enhancement | Technology | Benefit |
| --- | --- | --- |
| Real mains study lamp | 5 V relay module replacing LED | Practical lighting control for actual study use |
| Wi-Fi data logging | ESP8266 / ESP32 replacing or alongside Arduino | Cloud dashboard; email alerts if desk too hot |
| PIR motion sensor | HC-SR501 in parallel with HC-SR04 | Reduces false negatives when student sits very still |
| CO₂ / air quality sensor | MH-Z19 NDIR CO₂ sensor | Ventilation reminders for stuffy rooms |
| Mobile app interface | Bluetooth HC-05 + Android app | Remote monitoring and threshold configuration |

# Conclusion

The Smart Study Desk Assistant successfully demonstrates the integration of multiple sensors and output devices on an Arduino Uno platform to create a practical, energy-aware study environment controller. The system reliably detects student presence using ultrasonic ranging, automatically controls the study light, and continuously displays temperature and humidity data on an I²C LCD - all within a polling loop that executes every two seconds.

Testing confirmed that the HC-SR04 measured distances with errors consistently below 0.5 cm, and the DHT11 delivered readings within its stated accuracy specifications. All edge cases - sensor timeouts, DHT11 warm-up errors, and I²C address mismatches were identified during development and resolved through firmware guards and hardware verification..

# References

- Arduino LLC. (2024). Arduino Uno Rev3 - Datasheet & Documentation. Retrieved from https://docs.arduino.cc/hardware/uno-rev3

- Aosong Electronics Co., Ltd. (2020). DHT11 Product Datasheet (Rev 1.3).

- Cytron Technologies. (2022). HC-SR04 Ultrasonic Sensor User Guide. Retrieved from https://cytron.io

- NXP Semiconductors. (2021). I²C-Bus Specification and User Manual (Rev 7.0). UM10204.

- Adafruit Industries. (2024). DHT Sensor Library for Arduino. GitHub. https://github.com/adafruit/DHT-sensor-library

- de Brabander, F. (2023). LiquidCrystal_I2C Library. GitHub. https://github.com/fdebrabander/Arduino-LiquidCrystal-I2C-library

- Monk, S. (2022). Programming Arduino: Getting Started with Sketches (3rd ed.). McGraw-Hill Education.

- Margolis, M., Jepson, B., & Weldin, N. (2020). Arduino Cookbook (3rd ed.). O'Reilly Media.